# Lab: Routing

## Overview

Routing is a fundamental mechanic in the overall operations of not only networks but the internet at large. In today’s lab you’ll practice configuring BGP on two routers.

## Scenario

GlobeX needs to optimize its network traffic with its cloud infrastructure. You’ve been tasked with configuring BGP routing between the on-prem and cloud routers.

## Objectives

- Download the latest version of Cisco Packet Tracer (link can be found in the resources section below)

  - You will need to create an account and enroll in the “Getting Started with Cisco Packet Tracer” course to gain access to the download

- In packet tracer, configure BGP between two 2811 model routers

- For each router on your network, capture the routing table and configurations of the routing protocols

- Update your network topology diagram

## Resources

- Cisco Packet Tracer

- Packet Tracer - BGP Configuration

- Cisco IOS IP Routing: RIP Command Reference

- Static Routing Configuration

## Tasks

### Part 1: Staging

- Deploy two 2811 model routers in Packet Tracer

See diagram right below the instruction’s diagram.

  - Create a link between the two routers

  - Globex Edge Router

    - Interface FastEthernet0/0: BGP Link with IP address 156.12.1.5/30

    - Interface FastEthernet0/1: Globex internal network with network address 192.168.100.0/24 and gateway of 192.168.100.1

  - Cloud Edge Router

    - Interface FastEthernet0/0: BGP Link with IP address 156.12.1.6/30

    - Interface FastEthernet0/1: Cloud hosted network with network address 192.168.200.0/24 and gateway of 192.168.200.1

I know it says the neighbor is 156.12.1.6, I fixed that but forgot to screenshot it, It has been fixed in routing tables and pings.

- Each network should contain one 2960 model switch and at least one host

- Here is an example of what the network topology should look like:

### Part 2: Configure BGP Routing

- On the Globex router apply the following settings:

  - Enable BGP

  - Apply the following BGP configuration settings

    - Use private AS values

    - Set cloud router as a BGP neighbor

    - Share the LAN to the neighboring router using BGP

  - Capture screenshots of the data-populated screenshots of the FRR Status page including:

    - BGP Routes

    - BGP Summary

    - BGP Neighbors

- On the cloud router apply the following settings:

  - Enable BGP

  - Apply the following BGP configuration settings

    - Use private AS values

    - Set Globex router as a BGP neighbor

    - Share the LAN to the neighboring router using BGP

  - Capture screenshots of the data-populated screenshots of the router configurations:

    - BGP Routes

    - BGP Summary

    - BGP Neighbors

### Part 3: Routing Tables

- Locate and capture screenshots of the routing tables for:

  - Globex Router

Pings and Routing Table

  - Globex PC

IDK How to get a routing table for a PC, but here are some pings.

  - Cloud Router

Pings and Routing Table

  - Cloud-hosted PC

IDK How to get a routing table for a PC, but here are some pings.

- Capture screenshots of successful pings from the Globex PC to the cloud-hosted PC. Submit this as part of the assignment.